ARIA — Response Quality Report Mahindra BE6 · Blinking Warning Light Conversation · April 2026

| 2 / 7 Correct Responses | 5 / 7 Incorrect Responses | Overall Needs Significant Work |
| --- | --- | --- |

1.  Turn-by-Turn Response Quality

Each driver utterance is assessed against the BE6 PDF manual and web sources. Green = correct or acceptable. Red = incorrect or misleading.

| # | Driver Said | Verdict | ARIA Said | Should Have Said | Source |
| --- | --- | --- | --- | --- | --- |
| 1 | "...blinking light in the infotainment of my car. What can it be?" | ❌ Wrong | Guessed it was a charging system issue. Said to disconnect/reconnect the charger. | Should have noted that the infotainment screen is the centre touchscreen — not where warning telltales appear. Should ask: "Do you mean a light on your dashboard cluster, or something on the centre screen?" | BE6 Manual p.67–68 (CID section shows infotainment is a separate system from the instrument cluster) |
| 2 | "The light is blinking while driving. What are the possibilities?" | ✅ Correct | Asked for clarification: what color is it and where exactly (dashboard, charge port, or charger unit)? | This is exactly right. The manual supplement explicitly instructs ARIA to ask these two questions before guessing. | BE6 Manual Supplement — Complete Blinking Light Reference |
| 3 | "It is red warning light and it is flashing." | ✅ Correct | Said it is a serious/critical fault. Advised pulling over and calling Roadside Assistance at 1800 266 7070. | Correct. The manual confirms: red telltale while driving = CRITICAL. Stop as soon as safe. Call RSA 1800 266 7070. | BE6 Manual Supplement p.1 + Web (EV dashboard warning light standards) |
| 4 | "What can it possibly be? Have cool door now." | ❌ Wrong | Only responded to "cool" — activated remote cooling and said "just relax." Completely dropped the red warning light topic. | Should have answered the red light follow-up first: list possible causes (12V battery fault, general system fault, EPB fault, powertrain warning). Then briefly acknowledged the cooling request. | BE6 Telltale page (visual scan) — red indicators: battery, triangle, EPB, powertrain |
| 5 | "What do you think of the flashing red light? I have my car pulled over." | ❌ Wrong | Asked again: "is it a red warning light on your dashboard inside the car? What color is it?" | User already confirmed red + dashboard in Turn 3. Should have said: possible causes include 12V battery fault, general powertrain fault, or EPB fault. Do not restart. Wait for Mahindra RSA (1800 266 7070). Well done for pulling over. | BE6 Telltale visual scan + BE6 Manual p.72 (ABS lamp pattern applies to all telltales) |
| 6 | "Just red and it's on the dashboard on my car." | ❌ Wrong | Asked again: "Can you tell me which color it is — red, amber, green, blue, or white?" | Fourth time the user has confirmed red. Should have moved past clarification and listed specific possible red dashboard indicators: ⚠ General fault triangle, 🔋 12V battery, EPB circuit fault, powertrain warning. | BE6 Telltale page (visual analysis) + EV dashboard standards (web) |
| 7 | "I'm sitting inside my car, it is red and it is on the dashboard of my car." | ❌ Wrong | Asked again: "Can you tell me which color it is exactly?" | Should have given the definitive answer: red blinking dashboard light on BE6 = critical fault. Most likely causes: General system warning (red triangle), 12V battery failure, EPB fault, or powertrain/motor warning. Do NOT restart or drive. Stay in a safe location. RSA: 1800 266 7070. | BE6 Manual Supplement + BE6 Telltale visual analysis + Web (EV red light protocol) |

2.  Correct Answers — What ARIA Should Say

The following are the factually verified answers for the scenario in this conversation, confirmed from the BE6 PDF manual and web sources.

2a.  When the driver says "blinking light in the infotainment"

The infotainment screen (CID — Centre Information Display) is the touchscreen in the middle of the dashboard.

Warning telltale lights appear in the instrument cluster (DID — Driver Information Display), which is directly behind the steering wheel.

These are two different screens. A warning light cannot appear "in the infotainment."

Correct response: "Just to clarify — are you seeing a light in the cluster behind your steering wheel, or something on the centre touchscreen? Warning indicators only appear on the cluster."

2b.  When the driver says "blinking warning light" (unspecified)

There are four different locations where lights can blink on the BE6:

1. Dashboard telltale (instrument cluster, inside car) — indicates system faults

2. Charge port LED (rear-left flap of car) — Red blink = charge error, Amber blink = gun stuck

3. EVSE wall charger LED (on the charger unit, not the car) — White blink = system verification, Blue blink = scheduled charging

4. Exterior hazard lamps (outside the car) — number of flashes indicates lock/unlock/theft status

Correct response: Ask the driver to specify which location and what color.

2c.  When the driver confirms: red blinking light on the dashboard while driving

This is a CRITICAL warning. Confirmed by both the BE6 manual and EV industry standards.

Possible causes on the BE6 specifically (from telltale visual analysis of manual page 66):

• Red triangle ⚠ — General vehicle system fault (most common red telltale)

• Red battery rectangle — 12V auxiliary battery failure

• Red EPB circle — Electric Parking Brake circuit fault

• Red charging circle — Critical charging system fault

Correct response: "A red blinking light on your dashboard is serious and means a critical fault has been detected. The most likely causes are a battery fault, a general system warning, or a brake system issue. You have done the right thing by pulling over. Do not restart and drive further — call Mahindra Roadside Assistance now on 1800 266 7070. They are available 24/7 and toll-free."

2d.  When the driver asks "what can it possibly be?" as a follow-up

This is a continuation of the technical thread about the red light — it should not be treated as a new unrelated query.

ARIA must retain the context that the driver has already confirmed: red light, on dashboard, while driving, car now pulled over.

Correct response: "Based on what you described, the most likely possibilities on the BE6 are: a general system fault indicated by the red warning triangle, a 12V auxiliary battery fault, an Electric Parking Brake fault, or a high-voltage powertrain warning. Without a diagnostic scan it is impossible to pinpoint exactly which one. The right move is to stay put and call RSA at 1800 266 7070 — do not attempt to drive further."

3.  Where the Problems Are

Four distinct failure points were identified in this conversation.

| Problem | Where It Happens | Impact |
| --- | --- | --- |
| Infotainment vs. Instrument Cluster confusion | Turn 1 — intent classification does not distinguish CID (touchscreen) from DID (cluster) | ARIA guesses wrong system entirely and gives irrelevant charging advice |
| No conversation memory across turns | Turns 5, 6, 7 — the LLM re-asks clarifying questions already answered in Turn 3 | Driver has to repeat "red" and "dashboard" four times. Erodes trust completely |
| Follow-up questions misclassified as conversational | Turn 4: "What can it possibly be?" has no technical keywords, so manual is skipped | ARIA exits technical mode and responds to a side comment ("cool") instead |
| Whisper mishearing breaks intent | Turn 4: Whisper transcribes the driver's words as "Have cool door now" — "cool" triggers action mode | Entire red-light thread is abandoned; driver left without critical safety information |

4.  How to Improve

4a.  Separate infotainment from dashboard in the knowledge base

Add an explicit entry to be6_manual_supplement.txt: "The CID (Centre Information Display) is the touchscreen. Warning telltale lights only appear in the DID (Driver Information Display / instrument cluster behind the steering wheel). A warning light cannot appear in the infotainment screen." The intent classifier should also map the word 'infotainment' to ask a disambiguation question.

4b.  Strengthen conversation memory usage in the LLM prompt

The deque history already exists in the code (CONVERSATION_MEMORY = 4). The issue is the LLM (llama3:8b) is not reliably reading it. Add an explicit instruction at the top of every prompt: "IMPORTANT: If a prior turn shows the driver already answered a clarifying question, DO NOT ask it again. Use the answer they gave." Also bold or capitalise the history section in the prompt so the small model notices it.

4c.  Add follow-up phrases to the technical keyword list

Phrases like "what can it be", "what could it be", "what are the possibilities", "tell me more", "explain that", "what does that mean", "what now", "what should I do" should all trigger technical_query intent so the manual is searched and technical context is maintained across follow-up turns.

4d.  Add a context-carry guard for unresolved safety issues

If the last ARIA turn included the words "warning light", "fault", "critical", or "roadside assistance", treat the next turn as intent=mixed regardless of its own keywords. This prevents a misheard word like "cool" from erasing a live safety thread.

4e.  Hardcode the "red blinking dashboard = critical" response path

This should not rely on the LLM retrieving a RAG chunk. Once red + dashboard + driving is confirmed, ARIA should deterministically trigger a safety response: list the 4–5 possible causes from the BE6 telltale page, advise not restarting, and provide the RSA number 1800 266 7070. No guessing, no RAG retrieval needed.

5.  Golden Rule — Answer Correctly or State Capacity. Never Blunder.

The single most important principle for ARIA's reliability is this:

If ARIA knows the answer from the manual or from confirmed driver input → give the answer. If ARIA does not have enough information → ask ONE specific clarifying question, then wait. If ARIA cannot find the answer at all → say so clearly, and direct to RSA (1800 266 7070) or a Mahindra dealer.

What ARIA must never do:

✗  Guess a cause without confirming which light/location it is — leads to wrong safety guidance

✗  Ask the same clarifying question more than once — the driver already answered it

✗  Drop a safety topic because a side comment triggered a different intent

✗  Pretend to know the exact fault without a diagnostic scan — always acknowledge that certainty requires professional diagnosis

✗  Give an incorrect answer confidently — being wrong with confidence is worse than saying "I'm not sure, let me direct you to someone who can diagnose it"

What ARIA should always do for safety-critical questions:

✓  Acknowledge the driver's emotion first, then address the safety issue — both matter

✓  Give a specific, actionable answer based on the manual when the information is available

✓  Narrow down the possibilities (list 3–5 likely causes) rather than vague disclaimers

✓  Always end with a concrete next step: RSA number, dealer visit, or "do not restart the vehicle"

✓  Maintain the safety context across turns until it is resolved — do not let it be overwritten by other intents

Sources: BE6 Vehicle Manual (PDF) · BE6 Manual Supplement (be6_manual_supplement.txt) · EV Dashboard Warning Light Standards (web) · Visual analysis of BE6 Telltale page (manual p.66)